CS19541 - COMPUTER NETWORKS LAB

EXPERIMENT NO: 09

AIM

Implementation of SUBNETTING in CISCO PACKET TRACER simulator.

CREATING A NETWORK TOPOLOGY

Classless IP subnetting allows efficient utilization of IP address space by defining custom subnet masks. To construct the topology in Cisco Packet Tracer:

Open Cisco Packet Tracer and create a blank workspace by selecting New > Network > Generic.

Add Devices: Drag and drop 2 Routers (Router R1, Router R2), 2 Switches (Switch S1, Switch S2), and 20 PCs (5 PCs per LAN subnet) onto the canvas.

Interconnect Devices: Connect PCs to Switch access ports using Copper Straight-Through cables. Connect Switches to designated Router interfaces using Copper Straight-Through cables, and establish a WAN link between Router R1 and Router R2.

SUBNETTING & TOPOLOGY DIAGRAM

To subnet the network address 192.168.1.0/24 to accommodate at least 5 host addresses per network along with router/switch interfaces, a /27 subnet mask (255.255.255.224) is used:

Base Subnet Mask: 255.255.255.224 (/27)

Total Subnets Generated: 8 subnets

Usable Host Addresses per Subnet: 30 host addresses (2^5 - 2)

Network Topology Architecture:

[ LAN 1: 192.168.1.0/27 ] [ LAN 2: 192.168.2.0/27 ] PC1 - PC5 (192.168.1.11 - .15) PC1 - PC5 (192.168.2.11 - .15) \ ¦ / \ ¦ / [ Switch S1 ] [ Switch S1 ] ¦ ¦ Gig0/0 (192.168.1.1) Gig0/1 (192.168.2.1) +-------------------[ Router R1 ]------------------+ ¦ [ WAN Link ] ¦ +-------------------[ Router R2 ]------------------+ Fa0/0 (192.168.3.1) Fa0/1 (192.168.4.1) ¦ ¦ [ Switch S2 ] [ Switch S2 ] / ¦ \ / ¦ \ PC1 - PC5 (192.168.3.11 - .15) PC1 - PC5 (192.168.4.11 - .15) [ LAN 3: 192.168.3.0/27 ] [ LAN 4: 192.168.4.0/27 ]

DEVICE IP ADDRESSING SCHEME

| Device | Interface / Port | Assigned IP Address / Subnet | Default Gateway |
| --- | --- | --- | --- |
| Router R1 | GigabitEthernet0/0 | 192.168.1.1 /27 | N/A |
| Router R1 | GigabitEthernet0/1 | 192.168.2.1 /27 | N/A |
| Switch S1 (LAN 1) | FastEthernet0/1 (PC1 - PC5) | 192.168.1.11 - 192.168.1.15 | 192.168.1.1 |
| Switch S1 (LAN 2) | FastEthernet0/2 (PC1 - PC5) | 192.168.2.11 - 192.168.2.15 | 192.168.2.1 |
| Router R2 | FastEthernet0/0 | 192.168.3.1 /27 | N/A |
| Router R2 | FastEthernet0/1 | 192.168.4.1 /27 | N/A |
| Switch S2 (LAN 3) | FastEthernet0/1 (PC1 - PC5) | 192.168.3.11 - 192.168.3.15 | 192.168.3.1 |
| Switch S2 (LAN 4) | FastEthernet0/2 (PC1 - PC5) | 192.168.4.11 - 192.168.4.15 | 192.168.4.1 |

TESTING THE NETWORK

Network connectivity was verified using Cisco Packet Tracer simulation PDU transmission across devices and subnets. The test verification table is recorded below:

| Fire / Status | Last Status | Source | Destination | Type | Time (sec) | Periodic | Num |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal | Successful | PC4(2) | Router2 | ICMP | 0.000 | N | 12 |
| Normal | Successful | PC4(2) | PC2(1)(1) | ICMP | 0.000 | N | 13 |
| Normal | Successful | PC0 | Router0 | ICMP | 0.000 | N | 14 |

RESULT

The network topology was successfully designed, configured, and classless IP subnetting (/27) was implemented using the Cisco Packet Tracer simulator. Inter-subnet and intra-subnet communication between all hosts, switches, and routers was verified and tested successfully using ICMP ping packets.